Ordinance No 5 dated 11.06.2010 on minimum health and safety requirements regarding the exposure of workers to risks arising from physical agents (artificial optical radiation)

# Issued by the Minister of Labour and Social Policy and the Minister of Health, promulgated SG, Issue 49 dated 29.06.2010.

Section І

General

Article 1. This Ordinance determines:

1. the minimum requirements for protection of workers from existing or potential health and safety risks to eyes and skin arising from exposure to artificial optical radiation;

2. the exposure limit values for artificial non-coherent optical radiation, specified in Annex No 1;

3. the exposure limit values for laser (coherent) optical radiation, specified in Appendix No 2.

Article 2. This Ordinance applies together with the normative deeds pertaining to provision of healthy and safe labour conditions in observation of the more stringent specific and/or additional requirements, when defined by this Ordinance.

Section ІІ

Obligations of employers

Article 3. (1) In observation of requirements arising from Article 4, Article 16, Paragraph 1, Article 23, Paragraph 1, Article 26, Paragraph 1, Item 1 and Article 29 of the Health and Safety at Work Act (HSWA), in the event of exposure of workers to artificial optical radiation, the employer is obligated to assess and if required measure and/or calculate the probable levels of exposure in order to determine and apply the measures required for limiting the exposure to the respective limit values.

(2) Assessment of measurement and/or calculation of exposure levels are executed:

1. for laser radiation – in line with the national standards, implementing the standards of the International Electrotechnical Commission (IEC);

# 2. for non-coherent radiation – in line with the national standards and recommendations, implementing the standards and recommendations of the International Commission on Illumination (CIE) and the European Committee for Standartisation (CEN).

(3) In the event of exposure, not encompassed by the standards and recommendations pursuant to Paragraph 2 and until the adoption of such standards and recommendations, the assessment, measurement, and/or calculations shall be executed in line with approved methodologies, guides, and other national standards.

(4) The assessment of exposure levels to artificial optical radiation may be executed based upon data provided on part of the working/operating equipment manufacturers, when it conforms to the provisions of the Technical Requirements of Products Act and other normative deeds for assessment and certification of product conformity to the existing requirements.

Article 4. (1) The assessment, measurement, and/or calculation of exposure levels pursuant to the provisions of Article 3 must be planned and executed on part of the competent services and/or persons upon observation of the requirements pursuant to Article 217 of Ordinance No 7 dated 1999 on minimum requirements for healthy and safe labour conditions at the workplace and upon use of working equipment (promulgated SG, Issue 88 dated 1999), as well as pursuant to Article 26, 27, and 28 of HSWA.

(2) The data from the assessment, including the data from the measurement and/or calculation of exposure levels pursuant to Article 3 must be recorded and stored in a protocol.

Article 5. (1) The risk to health and safety of workers must be assessed pursuant to the provisions of Ordinance No 5 dated 1999 on the order, manner, and recurrence for execution of risk assessment (promulgated SG, Issue 47 dated 1999).

(2) During the risk assessment the employer must obligatorily take into account:

1. the level, spectral scope (wavelength range) and duration of exposure to artificial optical radiation;

2. the exposure limit values, referred to in Article 1, Item 2 and Item 3;

3. any effects concerning the health and safety of workers belonging to particularly sensitive risk groups;

4. any possible effects on workers’ health and safety resulting from workplace interactions between optical radiation and photosensitizing chemical substances;

5. any indirect effects such as temporary blinding, explosion or fire;

6. the existence of replacement equipment designed to reduce the levels of exposure to artificial optical radiation;

7. appropriate information obtained from the health surveillance, including published information, as far as possible;

8. multiple sources of exposure to artificial optical radiation at the workplace;

9. a classification applied to laser as defined pursuant to BDS EN 60825-1, and in relation to any artificial source likely to cause damage similar to that of class 3B or 4 lasers, and any similar classification;

10. information provided by the manufacturers of optical radiation sources and associated work equipment in accordance with the Technical Requirements of Products Act and other normative deeds for assessment and certification of conformity of products to the material requirements.

(3) The employer must document the risk assessment pursuant to the requirements of Ordinance No 5 dated 1999 on the order, manner, and recurrence of risk assessment execution.

(4) The employer is obligated to define the measures subject to undertaking pursuant to Article 6 and 7.

(5) The health and safety risk assessment shall be subjected to review pursuant to the order of Ordinance No 5 dated 1999 on the order, manner, and recurrence of risk assessment execution, as well as based upon the health surveillance results.

Article 6. (1) The employer is obligated to eliminate or reduce to a minimum the risks arising from exposure to artificial optical radiation, taking into account the technical progress and the presence of measures to control the risk at the source.

(2) The reduction of risks arising from exposure to artificial optical radiation shall be based upon the general principles of prevention, pursuant to the requirements of Article 4, Paragraph 3 of the Health and Safety at Work Act.

(3) Where the risk assessment carried out in accordance with Article 3 indicates the exposure limit values may be exceeded, the employer shall devise and implement a program of technical and /or organizational measures designed to prevent the exposure exceeding the limit values of artificial optical radiation, taking into account in particular:

1. other work methods, involving smaller risk from exposure to optical radiation;

2. the choice of equipment emitting less optical radiation, taking into account the work to be done;

3. technical measures to reduce the emission of optical radiation, including, where necessary, the use of interlocks, shielding or similar health protection mechanisms;

4. appropriate maintenance programs for work equipment, workplaces, and workstation systems;

5. the design and layout of workplaces and workstations;

6. limitation of the duration and level of exposure;

7. the availability of appropriate personal protective equipment;

8. the instructions of the equipment manufacturer, where it conforms to the provisions of the Technical Requirements of Products Act and the other normative deeds for assessment and certification of product conformity to the existing requirements.

(4) Workplaces for which the risk assessment pursuant to Article 3, 4, and 5 indicates that workers are or could be exposed to artificial optical radiation, exceeding the exposure limit values, shall be indicated with appropriate signs, pursuant to Ordinance No RD-07/8 dated 20.12.2008 on minimum requirements for signs and signals for safety and/or health during work (promulgated SG, Issue 3 dated 2009).

(5) The areas in question shall be identified pursuant to Paragraph 4 and access to said areas shall be limited, where this is technically possible, and where there is risk that the exposure limit values could be exceeded.

(6) Workers shall not be exposed above the exposure limit values pursuant to Article 1, Item 2 and 3.

(7) When exposure to artificial optical radiation is established exceeding the limit values, regardless of the measures adopted on part of the employer in observation of the requirements pursuant to the ordinance, the employer is obligated to:

1. undertake immediate action for reduction of exposure below the limit values;

2. determined the reasons leading to the exceeded limit values;

3. adapt undertaken measures for protection and prevention, in order to avoid subsequent exceeding of the limit values.

(8) The employer is obligated to adapt the measures pursuant to Paragraph 1, 2, 3, 4, 5, 6, and 7 to the workers particularly sensitive to exposure to artificial optical radiation.

Article 7. In observation of obligations arising from the Health and Safety at Work Act and Ordinance No RD-07-2 dated December 16th, 2009 on conditions and order for conduction of periodic training and instructions of workers and employees regarding the rules for provision of healthy and safe labour conditions (promulgated SG, Issue 102 dated 2009), the employer is obligated to provide workers, exposed to risks from artificial optical radiation during work and/or their representatives, required information and training, connected to the risk assessment results pursuant to Article 3, 4, and 5, in particular as regards to:

1. measures taken to implement the Ordinance’s requirements;

2. the exposure limit values and the associated potential risks;

3. the results of the assessment, measurement, and/or calculations of the levels of exposure to artificial optical radiation, pursuant to the provisions of Article 3, 4, and 5, together with explanation (clarifications) of their significance and potential risks;

4. the requirement for detection and reporting of unfavorable health effects resulting from the exposure;

5.  the circumstances in which workers are entitled to health surveillance;

6. safe working practices to minimize the risks from exposure to artificial optical radiation;

7. proper use of appropriate personal protective equipment.

Article 8. The employer is obligated to consult workers and/or their representatives and create conditions for their participation in all matters connected to the Ordinance’s application in line with the provisions of the Health and Safety at Work Act.

Section ІІІ

Health monitoring/surveillance

Article 9. The employer is obligated to provide health surveillance of workers, pursuant to the requirements of the Health and Safety at Work Act and Ordinance No 3 dated 1987 on mandatory preliminary and periodic medical examinations of workers (promulgated SG, Issue 16 dated 1987), for prevention and timely ascertainment of all adverse health effects and prevention of all long-term health risks and risks of chronic disease resulting from the exposure to optical radiation.

Article 10. (1) A current health file must be kept for each worker in line with the requirements of Ordinance No 3 dated 2008 on the conditions and order for implementation of activities of the occupational medicine services (promulgated SG, Issue 14 dated 2008), containing entries of recorded results of examinations and testing under this section.

(2) A copy of the file pursuant to Paragraph 1 must be provided upon request to the competent authorities, which possess the right of access to medical information and in line with the provisions of the Personal Data Protection Act.

(3) The employer must provide the occupational medicine service access to the risk assessment results pursuant to Article 3, 4, and 5.

(4) Access must be provided to worker’s file upon his/her request.

Article 11. (1) In the event, where exposure above the limit values is detected, a medical examination shall be made available to the worker/s by an ophthalmologist and dermatologist.

(2) The medical examination pursuant to Paragraph 1 must also be provided in the event of detected illness or unfavorable health effects to workers, which may be due to exposure to artificial optical radiation during work.

(3) In cases pursuant to Paragraph 1 and 2, when exceeding of exposure limit values is detected or unfavorable health effects, the physician or another medical person from the occupational medicine service must:

1. notify the worker regarding the results of the health surveillance personally pertaining to said worker;

2. provide information and advice regarding the health surveillance, conducted after the end of the exposure;

3. inform the employer taking into account any medical confidentiality.

Article 12. (1) In cases pursuant to Paragraph 1 and 2 of Article 11 the employer is obligated to:

1. review the risk assessment carried out pursuant to Article 3, 4, and 5;

2. review the measures provided for to eliminate or reduce risks pursuant to Article 6;

3. take into account the advice of the occupational health professional or other suitably qualified person or the competent control authority required to eliminate or reduce risk in accordance e with Article 6;

4. organize continued health surveillance and adopt measures for review of the health status of any other worker who has been similarly exposed.

(2) In cases pursuant to Paragraph 1, Item 4 the competent physician or occupational health professional from the occupational medicine service may propose that the exposed person undergo a medical examination, the employer organizes the conduction of a specialized medical examination.

Article 13. The Minister of health and the Minister of Labour and Social Policy, with the participation of the national representative organizations of workers and employers organize the development of a practical handbook to assist the implementation of the Ordinance’s requirements, in line with the recommended handbook of the European Commission for application of Directive 2006/25/EC.

ADDITIONAL REGULATIONS

§ 1. Within the meaning of this Ordinance:

1. “Optical radiation” shall mean any electromagnetic radiation in the wavelength range between 100 nm and 1 mm. The spectrum of optical radiation is divided into ultraviolet radiation, visible radiation, and infrared radiation:

a) “Ultraviolet radiation” shall mean optical radiation of wavelength range between 100 nm and 400 nm. The ultraviolet region is divided into: UVA (315 – 400 nm), UVB (280 - 315 nm), and UVC (100 - 280 nm);

b) “Visible radiation” shall mean optical radiation of wavelength range between 380 nm and 780 nm;

c) “Infrared radiation” shall mean optical radiation of wavelength range between 780 nm and 1 mm. The infrared region is divided into IRA (780 - 1400 nm), IRB (1400 - 3000 nm) and IRC (3000 nm – 1 mm);

2. “Laser” (light amplification by stimulated emission of radiation) shall mean any device, which can be made to produce or amplify electromagnetic radiation in the optical radiation wavelength range primarily by the process of controlled stimulated emission;

3. “Laser radiation” shall mean any optical radiation other than laser radiation;

4. “Non-coherent radiation” shall mean all optical radiation, save for laser radiation;

5. “Exposure limit values” shall mean exposure limits in exposure to optical radiation, which are based directly on established health effects and biological considerations. Compliance with these limits shall ensure that workers exposed to artificial sources of optical radiation are protected against all known adverse health effects;

6. “Incidence” (Е) or power density shall mean the radiant power incident per unit area upon a surface, expressed as watts per square meter, (Wm−2);

7. “Radiant exposure” (Н) the time integral of the irradiance, expressed in joules per square meter (Jm−2);

8. “Radiance” (L) shall mean the radiant flux or power output per unit solid angle per unit area (Wm−2sr−1);

9. “Level” shall mean the combination of irradiance, radiant exposure and radiance to which a worker is exposed.

§ 2. This Ordinance implements the provisions of Directive 2006/25/EC of the European Parliament and Council dated April 5th, 2006 on the minimum health and safety requirements regarding the exposure of workers to arising from physical agents (artificial optical radiation) (Nineteenth special directive within the meaning of Article 16, Paragraph 1 of Directive 89/391/EEC).

CONCLUDING REGULATIONS

§ 3. This Ordinance is issued based on the grounds of Article 36, Item 2 of the Health and Safety at Work Act.

§ 4. The handbook pursuant to Article 13 must be developed within a term up to December 30th, 2010.

- § 5. Ordinance No 8 dated 1986 on hygienic norms for laser radiation (promulgated SG, Issue 92 dated 1986) and Ordinance No 9 dated 1986 on sanitary rules for work with lasers (promulgated SG, Issue 95 dated 1986) are repealed.

Appendix No 1 to Article 1, Item 2

Non-coherent optical radiation

The biophysically relevant exposure values to optical radiation can be determined with the formulae below. The formulae to be used depend on the range of radiation emitted by the source and the results should be compared with the corresponding exposure limit values indicated in Table 1.1. More than one exposure value and the corresponding exposure limit can be relevant for a given source of optical radiation.

Numbering (1) to (15) refers to corresponding rows of Table 1.1.

(1) Heff=0tλ=180nmλ=400nmEλλ,t⋅Sλ⋅dλ⋅dt           Heff is only relevant in the range 180 to 400 nm;

(2) HUVA=0tλ=315nmλ=400nmEλλ,t⋅dλ⋅dt                   HUVA is only relevant in the range 315 to 400 nm;

(3), (4) LB=λ=300nmλ=700nmLλ(λ)⋅Bλ⋅dλ                Lλ is only relevant in the range 300 to 700 nm;

(5), (6) ЕB=λ=300nmλ=700nmЕλ(λ)⋅Bλ⋅dλ                ЕB is only relevant in the range 300 to 700 nm;

(7) to (12) LR=λ1λ2Lλλ⋅Rλ⋅dλ               See Table 1.1. for appropriate values of λ1and λ2;

(13), (14) ЕIR=λ=780nmλ=3000nmEλλ⋅dλ                    EIR is only relevant in the range 780 to 3000 nm;

(15) Hskin=0tλ=380nmλ=3000nmEλλ,t⋅dλ⋅dt                 Hskin is only relevant in the range 380 to 3000 nm.

For the purposes of the Ordinance the formulae above can be replaced by the following expressions and the use of discrete values as set out in the following table:

(1) Eeff=λ=180nmλ=400nmEλ⋅Sλ⋅Δλ                          and   Heff=Eeff⋅Δt

(2) EUVA=λ=315nmλ=400nmEλ⋅Δλ                                  and   HUVA=EUVA⋅Δt

(3), (4) LB=λ=300nmλ=700nmLλ⋅Bλ⋅Δλ

(5), (6) EB=λ=300nmλ=700nmEλ⋅Bλ⋅Δλ

(7) to (12) LR=λ1λ2Lλ⋅Rλ⋅Δλ                  See Table 1.1. for appropriate values of

λ1 andλ2;

(13), (14) EIR=λ=780nmλ=3000nmEλ⋅Δλ

(15) Eskin=λ=380nmλ=3000nmEλ⋅Δλ                               and   Hskin=Eskin⋅Δt

Notes:

Eλλ,t,Eλ   spectral irradiance or spectral power density (irradiance) – the radiant power incident per unit area, expressed in watts per square meter per nanometer [Wmˉ2nmˉ¹]; values of Eλλ,t,Eλ come from measurements or may be provided by the manufacturer of the equipment;

Eeff         	effective irradiance (UV range) calculated irradiance within the UV wavelength range 180 to 400 nm, spectrally weighted bySλ, expressed in watts per square meter [Wmˉ2];

H                	radiant exposure: the time integral of the irradiance, expressed in joules per square meter [Jmˉ2];

Heff                 effective radiant exposure – energy exposure spectrally weighted by Sλ, expressed in joules square meter [Jmˉ2];

EUVA                total irradiance (UVA) – calculated irradiance within the UVA wavelength range 315 to 400 nm, expressed in watts per square meter [Wmˉ2];

HUVA               irradiant exposure – the time of the wavelength integral or sum of the irradiance within the UVA wavelength range 315 to 400 nm, expressed in joules per square meter [Jmˉ2];

Sλ                spectral weighting, taking into account the wavelength dependence of the health effects of UV radiation on eyes and skin (Table 1.2), [dimensionless];

t,Δt                 time duration of the exposure expressed in seconds [s];

λ                     wavelength expressed in nanometers [nm];

Δλ                  bandwidth, expressed in nanometers [nm], of the calculation or measurement intervals;

Lλλ,Lλ        spectral radiance of the source expressed in watts per square meter per steradian per nanometer [Wm−2sr−1nm−1];

Rλ                spectral weighting, taking into account the wavelength dependence or the thermal injury caused to the eye by visible and IRA radiation (Table 1.3), [dimensionless];

LR                   effective radiance (thermal injury): calculated radiance spectrally weighted by Rλ, expressed in watts per square meter per steradian [Wm−2sr−1];

Bλ               spectral weighting, taking into account the wavelength dependence of the photochemical injury caused to the eye by blue light radiation (Table 1.3), [dimensionless];

LB                   effective radiance (blue light): calculated radiance spectrally weighted byBλ expressed in watts per square meter per steradian [Wm−2sr−1];

EB                   effective irradiance (blue light) calculated irradiance spectrally weighted by Bλ, expressed in watts per square meter [Wmˉ2];

EIR                  total irradiance (thermal injury): calculated irradiance within the infrared wavelength range 780 to 3000 nm, expressed in watts per square meter [Wmˉ2];

Eskin                total irradiance (visible, IRA and IRB) – calculated irradiance within the visible and infrared wavelength range  380 to 3000 nm, expressed in watts per square meter [Wmˉ2];

Hskin               radiant exposure – the time of the wavelength integral or sum of the irradiance within the visible and infrared wavelength range 380 to 3000 nm, expressed in joules per square meter [Jmˉ2];

α                    angular subtense – the angle subtended by an apparent source, as viewed at a point of space, expressed in milliradians [mrad]. Apparent source is the real or virtual object, which forms the smallest possible retinal image.

Table 1.1

Exposure limit values for non-coherent optical radiation

| Index |  | Wavelength nm |  | Exposure limit value |  | Units |  | Comment |  | Part of the body | Hazard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  | 180 – 400 (UVA, UVB and UVC) |  | Heff=30 Daily value 8 hours |  | [Jm−2] |  |  |  | Eye cornea Eye conjunctive sack Eye lens Skin | Photokeratitis Conjunctivitis Cataract (Cataractogenesis) Erythema Elastosis Skin cancer |
| 2. |  | 315 – 400 (UVA) |  | HUVA=104 Daily value 8 hours |  | [Jm−2] |  |  |  | Eye lens | Cataract (Cataractogenesis) |
| 3. |  | 300 – 700 (Blue light) see note 1 |  | LB=106t for t≤10000s |  | LB:[Wm−2sr−1] t:[s] |  | For α≥11mrad |  | Eye retina | Photoretinitis |
| 4. |  | 300 – 700 (Blue light) see note 1 |  | LB=100 for t>10000s |  | [Wm−2sr−1] |  |  |  |  |  |
| 5. |  | 300 – 700 (Blue light) see note 1 |  | EB=100t for t≤10000s |  | ЕB:[Wm−2] t:[s] |  | For α<11mrad see note 2 |  |  |  |
| 6. |  | 300 – 700 (Blue light) see note 1 |  | EB=0,01 t>10000s |  | [Wm−2] |  |  |  |  |  |
| 7. |  | 380 – 1400 (Visible and IRА) |  | LR=2,8⋅107Cα for t>10s |  | [Wm−2sr−1] |  | Cα=1,7 for α≤1,7mrad Cα=α for 1,7≤α≤100mrad Cα=100 for α>100mrad λ1=380; λ2=1400 |  | Eye retina | Retinal burn |
| 8. |  | 380 – 1400 (Visible and IRА) |  | LR=5⋅107Cαt0,25 for 10μs≤t≤10s |  | LR:[Wm−2sr−1] t: [s] |  |  |  |  |  |
| 9. |  | 380 – 1400 (Visible and IRА) |  | LR=8,89⋅108Cα for t<10μs |  | [Wm−2sr−1] |  |  |  |  |  |
| 10. | 780 – 1400 (IRА) |  | LR=6⋅106Cα for t>10s |  | [Wm−2sr−1] |  | Cα=11 for α≤11mrad Cα=α for 11≤α≤100mrad Cα=100 for α>100mrad (measured field-of-view: 11mrad) λ1=780; λ2=1400 |  | Eye retina |  | Retinal burn |
| 11. | 780 – 1400 (IRА) |  | LR=5⋅107Cαt0,25 for 10μs≤t≤10s |  | LR:[Wm−2sr−1] t: [s] |  |  |  |  |  |  |
| 12. | 780 – 1400 (IRА) |  | LR=8,89⋅108Cα for t<10μs |  | [Wm−2sr−1] |  |  |  |  |  |  |
| 13. | 780 – 3000 (IRА and IRВ) |  | ЕIR=18000t−0,75 for t≤1000s |  | Е: [Wm−2] t: [s] |  |  |  | Eye cornea lens |  | Corneal burn Cataractogenesis |
| 14. | 780 – 3000 (IRА and IRВ) |  | EIR=100 for t>1000s |  | [Wm−2] |  |  |  |  |  |  |
| 15. | 380 – 3000 (Visible, IRА and IRВ) |  | Hskin=20000t0,25 for t<10s |  | H:[Jm−2] t:[s] |  |  |  | Skin |  | Burn |
| Note 1 | The range of 300 to 700 nm covers parts of UVB, all UVA and most of visible radiation; however, the associated hazard is commonly referred to as „blue light”. Blue light strictly speaking covers only the range of approximately 400 to 490 nm. |  |  |  |  |  |  |  |  |  |  |
| - Note 2 | - For steady fixation of very small sources with angular subtense <11 mrad, LB can be converted toEB. This normally applies only to ophthalmic instruments or a stabilized eye during anesthesia. The maximum „stare time” is found by tmax=100EB, as EB expressed in Wm−2. Due to eye movements during normal visual tasks, it does not exceed 100 s. |  |  |  |  |  |  |  |  |  |  |

Table 1.2

S (λ) [dimensionless], 180 nm to 400 nm

| λ [nm] | S (λ) | λ [nm] | S (λ) | λ [nm] | S (λ) | λ [nm] | S (λ) | λ [nm] | S (λ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 180 | 0.0120 | 228 | 0.1737 | 276 | 0.9434 | 324 | 0.000520 | 372 | 0.000086 |
| 181 | 0.0126 | 229 | 0.1819 | 277 | 0.9272 | 325 | 0.000500 | 373 | 0.000083 |
| 182 | 0.0132 | 230 | 0.1900 | 278 | 0.9112 | 326 | 0.000479 | 374 | 0.000080 |
| 183 | 0.0138 | 231 | 0.1995 | 279 | 0.8954 | 327 | 0.000459 | 375 | 0.000077 |
| 184 | 0.0144 | 232 | 0.2089 | 280 | 0.8800 | 328 | 0.000440 | 376 | 0.000074 |
| 185 | 0.0151 | 233 | 0.2188 | 281 | 0.8568 | 329 | 0.000425 | 377 | 0.000072 |
| 186 | 0.0158 | 234 | 0.2292 | 282 | 0.8342 | 330 | 0.000410 | 378 | 0.000069 |
| 187 | 0.0166 | 235 | 0.2400 | 283 | 0.8122 | 331 | 0.000396 | 379 | 0.000066 |
| 188 | 0.0173 | 236 | 0.2510 | 284 | 0.7908 | 332 | 0.000383 | 380 | 0.000064 |
| 189 | 0.0181 | 237 | 0.2624 | 285 | 0.7700 | 333 | 0.000370 | 381 | 0.000062 |
| 190 | 0.0190 | 238 | 0.2744 | 286 | 0.7420 | 334 | 0.000355 | 382 | 0.000059 |
| 191 | 0.0199 | 239 | 0.2869 | 287 | 0.7151 | 335 | 0.000340 | 383 | 0.000057 |
| 192 | 0.0208 | 240 | 0.3000 | 288 | 0.6891 | 336 | 0.000327 | 384 | 0.000055 |
| 193 | 0.0218 | 241 | 0.3111 | 289 | 0.6641 | 337 | 0.000315 | 385 | 0.000053 |
| 194 | 0.0228 | 242 | 0.3227 | 290 | 0.6400 | 338 | 0.000303 | 386 | 0.000051 |
| 195 | 0.0239 | 243 | 0.3347 | 291 | 0.6186 | 339 | 0.000291 | 387 | 0.000049 |
| 196 | 0.0250 | 244 | 0.3471 | 292 | 0.5980 | 340 | 0.000280 | 388 | 0.000047 |
| 197 | 0.0262 | 245 | 0.3600 | 293 | 0.5780 | 341 | 0.000271 | 389 | 0.000046 |
| 198 | 0.0274 | 246 | 0.3730 | 294 | 0.5587 | 342 | 0.000263 | 390 | 0.000044 |
| 199 | 0.0287 | 247 | 0.3865 | 295 | 0.5400 | 343 | 0.000255 | 391 | 0.000042 |
| 200 | 0.0300 | 248 | 0.4005 | 296 | 0.4984 | 344 | 0.000248 | 392 | 0.000041 |
| 201 | 0.0334 | 249 | 0.4150 | 297 | 0.4600 | 345 | 0.000240 | 393 | 0.000039 |
| 202 | 0.0371 | 250 | 0.4300 | 298 | 0.3989 | 346 | 0.000231 | 394 | 0.000037 |
| 203 | 0.0412 | 251 | 0.4465 | 299 | 0.3459 | 347 | 0.000223 | 395 | 0.000036 |
| 204 | 0.0459 | 252 | 0.4637 | 300 | 0.3000 | 348 | 0.000215 | 396 | 0.000035 |
| 205 | 0.0510 | 253 | 0.4815 | 301 | 0.2210 | 349 | 0.000207 | 397 | 0.000033 |
| 206 | 0.0551 | 254 | 0.5000 | 302 | 0.1629 | 350 | 0.000200 | 398 | 0.000032 |
| 207 | 0.0595 | 255 | 0.5200 | 303 | 0.1200 | 351 | 0.000191 | 399 | 0.000031 |
| 208 | 0.0643 | 256 | 0.5437 | 304 | 0.0849 | 352 | 0.000183 | 400 | 0.000030 |
| 209 | 0.0694 | 257 | 0.5685 | 305 | 0.0600 | 353 | 0.000175 |  |  |
| 210 | 0.0750 | 258 | 0.5945 | 306 | 0.0454 | 354 | 0.000167 |  |  |
| 211 | 0.0786 | 259 | 0.6216 | 307 | 0.0344 | 355 | 0.000160 |  |  |
| 212 | 0.0824 | 260 | 0.6500 | 308 | 0.0260 | 356 | 0.000153 |  |  |
| 213 | 0.0864 | 261 | 0.6792 | 309 | 0.0197 | 357 | 0.000147 |  |  |
| 214 | 0.0906 | 262 | 0.7098 | 310 | 0.0150 | 358 | 0.000141 |  |  |
| 215 | 0.0950 | 263 | 0.7417 | 311 | 0.0111 | 359 | 0.000136 |  |  |
| 216 | 0.0995 | 264 | 0.7751 | 312 | 0.0081 | 360 | 0.000130 |  |  |
| 217 | 0.1043 | 265 | 0.8100 | 313 | 0.0060 | 361 | 0.000126 |  |  |
| 218 | 0.1093 | 266 | 0.8449 | 314 | 0.0042 | 362 | 0.000122 |  |  |
| 219 | 0.1145 | 267 | 0.8812 | 315 | 0.0030 | 363 | 0.000118 |  |  |
| 220 | 0.1200 | 268 | 0.9192 | 316 | 0.0024 | 364 | 0.000114 |  |  |
| 221 | 0.1257 | 269 | 0.9587 | 317 | 0.0020 | 365 | 0.000110 |  |  |
| 222 | 0.1316 | 270 | 1.0000 | 318 | 0.0016 | 366 | 0.000106 |  |  |
| 223 | 0.1378 | 271 | 0.9919 | 319 | 0.0012 | 367 | 0.000103 |  |  |
| 224 | 0.1444 | 272 | 0.9838 | 320 | 0.0010 | 368 | 0.000099 |  |  |
| 225 | 0.1500 | 273 | 0.9758 | 321 | 0.000819 | 369 | 0.000096 |  |  |
| 226 | 0.1583 | 274 | 0.9679 | 322 | 0.000670 | 370 | 0.000093 |  |  |
| 227 | 0.1658 | 275 | 0.9600 | 323 | 0.000540 | 371 | 0.000090 |  |  |

Table 1.3

B (λ), R (λ) [dimensionless], 380 nm to 1400 nm

| λ [nm] | B (λ) | R (λ) |
| --- | --- | --- |
| 300 ≤ λ <380 | 0,01 | - |
| 380 | 0,01 | 0,1 |
| 385 | 0,013 | 0,13 |
| 390 | 0,025 | 0,25 |
| 395 | 0,05 | 0,5 |
| 400 | 0,1 | 1 |
| 405 | 0,2 | 2 |
| 410 | 0,4 | 4 |
| 415 | 0,8 | 8 |
| 420 | 0,9 | 9 |
| 425 | 0,95 | 9,5 |
| 430 | 0,98 | 9,8 |
| 435 | 1 | 10 |
| 440 | 1 | 10 |
| 445 | 0,97 | 9,7 |
| 450 | 0,94 | 9,4 |
| 455 | 0,9 | 9 |
| 460 | 0,8 | 8 |
| 465 | 0,7 | 7 |
| 470 | 0,62 | 6,2 |
| 475 | 0,55 | 5,5 |
| 480 | 0,45 | 4,5 |
| 485 | 0,32 | 3,2 |
| 490 | 0,22 | 2,2 |
| 495 | 0,16 | 1,6 |
| 500 | 0,1 | 1 |
| 500 < λ ≤ 600 | 100,02⋅450−λ | 1 |
| 600 < λ ≤ 700 | 0,001 | 1 |
| 700 < λ ≤ 1050 | - | 100,002⋅700−λ |
| 1050 < λ ≤ 1150 | - | 0,2 |
| 1150 < λ ≤ 1200 | - | 0,2⋅100,02⋅1150−λ |
| 1200 < λ ≤ 1400 | - | 0,02 |

Appendix No 2 to Article 1, Item 3

Laser optical radiation

The biophysically relevant exposure values to optical radiation can be determined by the formulae below. The formulae to be used depend on the wavelength and duration of radiation emitted by the source and the results should be compared with the corresponding exposure limit values indicated in Table 2.2 to 2.4. More than one exposure value and corresponding exposure limit can be relevant for a given source of laser optical radiation.

Coefficients used as calculation tools within Tables 2.2 to 2.4 are listed in Table 2.5, and corrections for repetitive exposure are listed in Table 2.6.

Е=dPdAWm−2

H=0tEt⋅dtJm−2

Notes:

dP                  power, expressed in watt [W];

dA                  surface, expressed in square meters [m2];

Et, E	irradiance (power density) – the irradiant power, incident per unit area upon a surface, generally expressed in watts per square meter [Wm−2]. Values of Et, E come from measurements or may be provided by the manufacturer of the equipment;

H                   radiant exposure  - the time integral of the radiance, expressed in joules per square meter [Jmˉ2 ];

t                     time, duration of exposure, expressed in seconds [s];

λ                   wavelength, expressed in nanometers [nm];

γ                    limiting cone angle of measurement of field-of-view, expressed in milliradiansmrad;

γm                  measuring field of view, expressed in milliradians mrad;

α                   angular subtense of a source, expressed in milliradians mrad;

limiting aperture – the circular area, over which irradiance and radiant exposure are averaged;

G             integrated radiance: the integral of the radiance over a given exposure time expressed as radiant energy per unit area of a radiating surface per unit solid angle of emission, in joules per square meter per steradian [Jm−2st−1].

Table 2.1

Radiation hazards

| Wavelength [nm] λ | Radiation range | Affected organ | Hazard | Exposure limit value table |
| --- | --- | --- | --- | --- |
| from 180 to 400 | UV | Eye | Photochemical damage and thermal damage | 2.2, 2.3 |
| from 180 to 400 | UV | Skin | Erythema | 2.4 |
| from 400 to 700 | Visible | Eye | Retinal damage | 2.2 |
| from 400 to 600 | Visible | Eye | Photochemical damage | 2.3 |
| from 400 to 700 | Visible | Skin | Thermal damage | 2.4 |
| from 700 to 1400 | IRA | Eye | Thermal damage | 2.2, 2.3 |
| from 700 to 1400 | IRA | Skin | Thermal damage | 2.4 |
| from 1400 to 2600 | IRB | Eye | Thermal damage | 2.2 |
| from 2600 to 10⁶ | IRC | Eye | Thermal damage | 2.2 |
| from 1400 to 10⁶ | IRB, IRC | Eye | Thermal damage | 2.3 |
| from 1400 to 10⁶ | IRB, IRC | Skin | Thermal damage | 2.4 |

Table 2.2

Exposure limit values for laser exposure to the eye – Short exposure duration < 10s

| Wavelength a, [nm] |  | Aperture | Duration [s] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10−13−10−11 | 10−11−10−9 |  | 10−9−10−7 | 10−7−1,8.10−5 | 1,8.10−5−5.10−5 | 5.10−5−10−3 | 10−3−101 |
| UVC | 180-280 | 1 mm for t < 0,3 s; 1,5 x t ⁰’³⁷⁵ for 0,3 < t < 10 s | Е=3x1010, [Wm−2] See note c |  |  | H=30[Jm−2] |  |  |  |  |
| UVB | 280-302 |  |  |  |  |  |  |  |  |  |
|  | 303 |  |  |  |  | H=40[Jm−2]; if t<2,6.10−9, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 304 |  |  |  |  | H=60[Jm−2]; if t<1,3.10−8, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 305 |  |  |  |  | H=100[Jm−2]; if t<1,0.10−7 then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 306 |  |  |  |  | H=160[Jm−2]; if t<6,7.10−7, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 307 |  |  |  |  | H=250[Jm−2]; if t<4,0.10−6, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 308 |  |  |  |  | H=400[Jm−2]; if t<2,6.10−5, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 309 |  |  |  |  | H=630[Jm−2]; if t<1,6.10−4, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 310 |  |  |  |  | H=103[Jm−2]; if t<1,0.10−3, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 311 |  |  |  |  | H=1,6.103[Jm−2]; if t<6,7.10−3, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 312 |  |  |  |  | H=2,5.103[Jm−2]; if t<4,0.10−2, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 313 |  |  |  |  | H=4,0.103[Jm−2]; if t<2,6.10−1, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
|  | 314 |  |  |  |  | H=6,3.103[Jm−2]; if t<1,6.100, then H=5,6.103t0,25 [Jm−2], See note d |  |  |  |  |
| UVA | 315-400 |  |  |  |  | H=5,6.103t0,25 [Jm−2] |  |  |  |  |
| Visible and IRA | 400-700 | 7 mm | H=1,5.10−4CE [Jm−2] |  | H=2,7.104t0,75CE [Jm−2] | H=5.10−3.CE [Jm−2] |  | H=18.t0,75CE [Jm−2] |  |  |
|  | 700-1050 |  | H=1,5.10−4CACE [Jm−2] |  | H=2,7.104t0,75CECA [Jm−2] | H=5.10−3.CACE [Jm−2] |  | H=18.t0,75CACE [Jm−2] |  |  |
|  | 1050-1400 |  | H=1,5.10−4CСCE [Jm−2] |  | H=2,7.105t0,75CCCE [Jm−2] | H=5.10−2.CCCE [Jm−2] |  |  | H=90.t0,75CCCE[Jm−2] |  |
| IRB and IRC | 1400-1500 | See note c | Е=1012, [Wm−2], See note c |  |  | H=103 [Jm−2] |  |  | H=5,6.103t0,25 [Jm−2] |  |
|  | 1500-1800 |  | Е=1013, [Wm−2], See note c |  |  | H=104 [Jm−2] |  |  |  |  |
|  | 1800-2600 |  | Е=1012, [Wm−2], See note c |  |  | H=103 [Jm−2] |  |  | H=5,6.103t0,25 [Jm−2] |  |
|  | 2600-106 |  | Е=1011, [Wm−2], See note c |  |  | H=100[Jm−2] | H=5,6.103t0,25 [Jm−2] |  |  |  |

a - If the wavelength of the laser is covered by two limits, then the more restrictive applies;

b - When1400≤λ<105nm, aperture diameter = 1 mm for t≤0,3s и 1,5t0,375mm for 0,3s<t<10s; when 105≤λ<106nm: aperture diameter = 11 mm;

c - Due to lack of data for these pulse lengths, ICNIRP recommends the use of the 1 ns irradiance limits;

d - The table states values for single laser pulses. In case of multiple laser pulses, then the laser pulse durations falling within the interval Tmin (listed in Table 2.6) must be added up in the formula 5,6.103t0,25 and the resulting time value must be filled in for t in the formula.

Table 2.3

Exposure limit values for laser exposure to the eye – long exposure duration  10s

| Wavelength a, [nm] |  | Aperture | Duration [s] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 101−102 | 102−104 | 104−3.104 |
| UVC | 180-280 | 3,5 mm | H=30[Jm−2] |  |  |
| UVB | 280-302 |  |  |  |  |
|  | 303 |  | H=40[Jm−2] |  |  |
|  | 304 |  | H=60[Jm−2] |  |  |
|  | 305 |  | H=100[Jm−2] |  |  |
|  | 306 |  | H=160[Jm−2] |  |  |
|  | 307 |  | H=250[Jm−2] |  |  |
|  | 308 |  | H=400[Jm−2] |  |  |
|  | 309 |  | H=630[Jm−2] |  |  |
|  | 310 |  | H=1,0.103[Jm−2] |  |  |
|  | 311 |  | H=1,6.103[Jm−2] |  |  |
|  | 312 |  | H=2,5.103[Jm−2] |  |  |
|  | 313 |  | H=4,0.103[Jm−2] |  |  |
|  | 314 |  | H=6,3.103[Jm−2] |  |  |
| UVA | 315-400 |  | H=104[Jm−2] |  |  |
| Visible 400 - 700 | 400 – 600 Photochemical b Retinal damage | 7 mm | H=100CB [Jm−2] (γ=11mrad)d | Е=1CB [Wm−2]; 1,1t0,5mradd | E=1CB [Wm−2] (γ=110mrad)d |
|  | 400-700 Thermal b Retinal damage |  | if α<1,5mrad, then E=10 [Wm−2] if α>1,5mrad and t≤T2, then H=18CEt0,75 [Jm−2] if α>1,5mrad and t>T2, then E=18CET2−0,25 [Wm−2] |  |  |
| IRA | 700 - 1400 | 7 mm | if α<1,5mrad, then E=10CACC [Wm−2] if α>1,5mrad and t≤T2, then H=18CACCCEt0,75 [Jm−2] if α>1,5mrad and t>T2, then E=18CACCCET2−0,25 [Wm−2] (not to exceed 1000 Wm−2) |  |  |
| IRB and IRC | 1400 - 106 | See c | Е=1000 [Wm−2] |  |  |

a – If the wavelength or another condition of the laser is covered by two limits, then the more restrictive applies;

b – For small sources subtending an angle of 1,5 mrad or less the visible dual limits E from 400 nm to 600 nm reduce to the thermal limits for 10s≤t<T1 and to photochemical limits for longer times. For T1 and T2 see Table 2.5. The photochemical retinal hazard limit may also be expressed as a time integral radiance G=106CB [Jm−2sr−1] for t>10s up to t=10000 s and L=100CB [Wm−2sr−1] for t>10000s. For the measurement of G and L, γm must be used average field of view. The official border between visible and infrared as defined by CIE is 780 nm. The column with wavelength band names is only meant to provide better overview for the user. The notation G is used by CEN, the notationLt is used by CIE, the notation Lpis used by IEC and CENELEC;

c – For wavelength 1400−103nm: aperture diameter = 3,5 mm; for wavelength 105−106nm: aperture diameter = 11 mm;

d – For measurement of the exposure value, the consideration of γ is defined as follows: If α(angular subtense of a source)γ(limiting cone angle, indicated in brackets in the corresponding column), then the measurement field of view γm should be the given value of γ (If a larger measurement field of view is used, then the hazard would be overestimated).

If α<γ, then the measurement field of view must be large enough to fully enclose the source, but is otherwise not limited and may be larger thanγ.

Table 2.4

Exposure limit values for laser exposure of skin

| Wavelength a, [nm] |  | Aperture | Duration [s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10−9 | 10−9−10−7 | 10−7−10−3 | 10−3−101 | 101−103 | 103−3.104 |
| UV (A, B, C) | 180 - 400 | 3,5 mm | E=3.1010 [Wm−2] | Same as eye exposure limits |  |  |  |  |
| Visible and IRA | 400 - 700 | 3,5 mm | E=2.1011 [Wm−2] | H=200CA | H=1,1.104CAt0,25 [Jm−2] |  | Е=2.103CA [Wm−2] |  |
|  | 700 - 1400 |  | E=2.1011CA [Wm−2] | [Jm−2] |  |  |  |  |
| IRB and IRC | 1400 - 1500 |  | E=1012 [Wm−2] | Same as eye exposure limits |  |  |  |  |
|  | 1500 - 1800 |  | E=1013 [Wm−2] |  |  |  |  |  |
|  | 1800 - 2600 |  | E=1012 [Wm−2] |  |  |  |  |  |
|  | 2600 - 106 |  | E=1011 [Wm−2] |  |  |  |  |  |

a  - If the wavelength or another condition of the laser is covered by two limits, then the more restrictive applies.

Table 2.5

Applied correction factors and other calculation parameters

| Parameter as listed in ICNIRP | Valid spectral range [nm] | Value |
| --- | --- | --- |
| CA | λ<700 |  |
|  | 700 - 1050 | CA=100,002λ−700 |
|  | 1050 - 1400 | CA=5,0 |
| CB | 400 - 450 | CB=1,0 |
|  | 450 - 700 | CB=100,02λ−450 |
| CC | 700 - 1150 | CC=1,0 |
|  | 1150 - 1200 | CC=100,018λ−1150 |
|  | 1200 - 1400 | CC=8,0 |
| T1 | λ<450 | T1=10s |
|  | 450 - 500 | T1=10⋅100,02λ−450s |
|  | λ>500 | T1=100s |
| Parameter as listed in ICNIRP | Valid for biological effect | Value |
| αmin | All thermal effects | αmin=1,5mrad |
| Parameter as listed in ICNIRP | Valid angular range (mrad) | Value |
| CE | α<αmin | CE=1,0 |
|  | αmin<α<100 | CE=ααmin |
|  | α>100 | CE=α2αmin⋅αmaxmrad с αmax=100mrad |
| T2 | α<1,5 | T2=10s |
|  | 1,5<α<100 | T2=10⋅10α−1,598,5s |
|  | α>100 | T2=100s |
| γ | t≤100 | γ=11mrad |
|  | 100<t<104 | γ=1,1⋅t0,5mrad |
|  | t>104 | γ=110mrad |

Table 2.6

Correction for repetitive exposure

(pulse radiation and scanning lasers)

Each of the following three general rules should be applied to all repetitive exposures occurring from repetitively pulsed or scanning laser systems:

1. The exposure from any single pulse in a train of pulses should not exceed the exposure limit value for a single pulse of that pulse duration.

2. The exposure from any group of pulses or sub-group of pulses in a train), delivered in time t, shall not exceed the exposure limit value for time t.

3. The exposure from any single pulse within a group of pulses shall not exceed the single-pulse exposure limit value multiplied by a cumulative-thermal correction factorCp=N−0,25, where N is the number of pulses. This rule applies only to exposure limits to protect against thermal injury, where all pulses delivered in less than Tmin are treated as a single pulse.

| Parameter | Valid spectral range, [nm] | Value |
| --- | --- | --- |
| Tmin | 315<λ≤400 | Tmin=10−9s1ns |
|  | 400<λ≤1050 | Tmin=18⋅10−6s18μs |
|  | 1050<λ≤1400 | Tmin=50⋅10−6s50μs |
|  | 1400<λ≤1500 | Tmin=10−3s1ms |
|  | 1500<λ≤1800 | Tmin=10s |
|  | 1800<λ≤2600 | Tmin=10−3s1ms |
|  | 2600<λ≤106 | Tmin=10−7s100ns |